MASONRY

MODULE 1

## CON/OS/MSN/CR/01/3/MA

## CONSTRUCT MASONRY STRUCTURES

September/ November 2025

TVET CURRICULUM DEVELOPMENT, ASSESSMENT AND CERTIFICATION COUNCIL (TVET CDACC)

PRACTICAL ASSESSMENT

INSTRUCTIONS TO THE ASSESSOR:

- This assessment is to take place in the prescribed order as arranged in the tool.

- Capture clear photographs and/or videos of each candidate’s work at critical points as they perform the tasks and label all media files with: Candidate Registration Number, Unit Code, Practical Session Number, and Date.

- Record candidate scores and assessor remarks in the observation checklists for each session.

- Store all completed checklists, media files, and candidate drawings in a secure digital/physical folder per candidate.

CANDIDATE & ASSESSOR DETAILS

| Candidate Name: | CDACC Reg. No.: |
| --- | --- |
| Assessor Name: | Assessor ID Number: |

PRACTICAL BRIEF

In this practical, you will be required to demonstrate competence in constructing masonry structures based on the provided drawing.  The assessment will involve hands-on sessions and an oral assessment.

SESSION 1 (4 HOURS): STUDY AND INTERPRET THE DRAWING, SET OUT THE BUILDING USING 3 4 5 METHOD, ERECT PROFILE BOARDS AND MARK OUT THE TRENCHES ON THE GROUND USING LIME.

| Assessment Date: |  |
| --- | --- |
| Assessment Venue: |  |

PRACTICAL CHECKLIST

| No. | Items of Evaluation | Max Marks | Awarded |
| --- | --- | --- | --- |
|  | Donned personal protective gears according to Standard Operating Procedures. - Overall. - Safety boots. - Helmet. (Award 1 mark for each PPE worn) | 3 |  |
|  | Interpreted the working drawing. (Award 2 marks for correct interpretation) | 2 |  |
|  | Marked the overall length and width on the ground by use of pegs. - 4000 x 2 - 3700x 2 (Award 1 mark for each measurement.) | 4 |  |
|  | Used 3-4-5 method to set out the whole outline. (Award 1 mark each line) | 4 |  |
|  | Fixed six profile boards as per the building plan. (Award 3 marks for each profile boards properly erected) | 18 |  |
|  | Marked points with saw cut or nails on the profile board - 200 mark - 600 mark (Award 1 mark each, for any two adjacent profile boards) | 2 |  |
|  | Fixed builder lines in correct position as per the working drawing. (Award 1 mark for each side 1x4) | 4 |  |
|  | Marked the position of the trench with lime. (Award 1 Mark for each side 6x1 =6 Marks) | 6 |  |
|  | Checked the diagonals of the plan Award 2 marks for each diagonal correctly checked ) | 4 |  |
|  | Dimensions. - 1500 - 2000 - 2500 (Award 1 mark for each) | 3 |  |
|  | Total section | 50 |  |
| REMARKS: |  |  |  |

NB: Photos and videos should be taken as the candidate performs items 4, 5, 7, 8 and 9.

SESSION 2 (12 HOURS): EXCAVATE FOUNDATION TRENCH 500 MM DEEP, LAY FOUNDATION CONCRETE (1:3:6) IN THE TRENCH BOTTOM, CONSTRUCT FOUNDATION WALLING AND CONSTRUCT SOLID GROUND FLOOR SLAB.

| Assessment Date: |  |
| --- | --- |
| Assessment Venue: |  |

PRACTICAL CHECKLIST

| No. | Items of Evaluation | Max Marks | Awarded |
| --- | --- | --- | --- |
|  | Donned personal protective gears according to Standard Operating Procedures. - Overall. - Safety boots. - Helmet. (Award 1 mark for each PPE worn) | 3 |  |
|  | Interpreted the working drawing correctly. (Award 2 marks for correct interpretation) | 2 |  |
|  | Assembled and used tools as per the manufacturer’s instructions. - Plumb bob - Trowel - Wooden float (Award 1 mark each tool) | 3 |  |
|  | Set out and marked the trenches according to the building plan. (Award 1 mark for each side of the trench) | 4 |  |
|  | Excavated trenches to a consistent depth of 500mm deep as per the working drawing. (Award 1 mark for each trench) | 4 |  |
|  | Leveled the trench bottom to receive foundation blinding. - Uniform level - Removal of Loose Material - Firm bottom (Award 1 mark for each) | 3 |  |
|  | Mixed foundation concrete in the correct ratio 1: 3: 6 - Correct proportion - Even mix (Award 1 mark for each) | 2 |  |
|  | Poured and compacted the foundation concrete evenly across trench bottom. - Leveling pegs - Wetting the bottom - Smooth concrete surface (Award 1 mark for each procedure) | 3 |  |
|  | Mixed mortar in the ratio of 1:3 (Award 3 marks for correct procedure) | 3 |  |
|  | Set out the wall on ground as per the drawing provided (Award 3 marks for correct procedure) | 3 |  |
|  | Laid and constructed course 1 and 3 correctly (Award 2 x 2 marks for each laid course = 4 Marks) | 4 |  |
|  | Laid and constructed course 2 correctly (Award 2 marks) | 2 |  |
|  | Dimensions (mm) - 1250 - 820 - 750 (Award 1 Mark each) | 3 |  |
|  | Joint - Uniformity - Thickness - Levelness (Award 3 x 1 mark for each =3 Marks) | 3 |  |
|  | Wall plumbness (Award 1mark each for any 4 places) | 4 |  |
|  | Squareness at the four corners - Bottom - Middle - Top (Award 1mark for each) | 3 |  |
|  | Packed Hardcore (Award 2 marks for correct hard packing and 1 mark for correct consolidation) | 3 |  |
|  | Laid Murram (Award 1 marks for correct laying and 1 mark for correct consolidation) | 2 |  |
|  | Laid DPM (Award 2 marks for correct procedure) |  |  |
|  | Mixed and casted concrete. - Batching - Mixing - Casting - Compaction (Award 1 mark each) | 4 |  |
|  | TOTAL | 60 |  |
| REMARKS: |  |  |  |

NB: Photos and videos should be taken as the candidate performs items 4, 5, 6, 7, 8, 9, 10, 11, 12, 17, 18, 19 and 20.

SESSION 3 (8 HOURS): MAKE MORTAR (1:3), CONSTRUCT SUPERSTRUCTURE WALL ACCORDING TO THE WORKING DRAWING AND CONSTRUCT A REINFORCED CONCRETE LINTEL OF 800X 150 X 200MM.

| Assessment Date: |  |
| --- | --- |
| Assessment Venue: |  |

PRACTICAL CHECKLIST

| No. | Items of Evaluation | Max Marks | Awarded |
| --- | --- | --- | --- |
|  | Donned personal protective gears according to Standard Operating Procedures. - Overall. - Safety boots. - Helmet. (Award 1 mark for each PPE worn) | 3 |  |
|  | Interpreted the working drawing correctly. (Award 2 marks for correct interpretation) | 2 |  |
|  | Set out the wall as per working drawings (Award 2 marks for setting out) | 2 |  |
|  | Batched mortar at a ratio of 1:3 (Award 2 marks) | 2 |  |
|  | Mixed mortar to workable consistency (Award 2 marks for uniform mix) | 2 |  |
|  | Laid a bed of mortar to accommodate the length of the first course. (Award 1 mark) | 1 |  |
|  | Laid and constructed course 1 and 3 correctly (Award 2 marks for each correctly laid course) | 4 |  |
|  | Laid and constructed course 2 and 4 correctly (Award 2 marks for each correctly laid course) | 4 |  |
|  | Filled the joints. (Award 1 mark for each course filled proper joint for the four courses) | 4 |  |
|  | Prepared the concrete mix for the lintel (1:2:4) (Award 2 marks for proportion and 1 mark for uniformity) | 3 |  |
|  | Cut four horizontal bars to the length of 800mm as per the drawing. (Award 1 mark for each bar) | 4 |  |
|  | Bended the three stirrups (Award 2 marks for each stirrup) | 6 |  |
|  | Fixed the three stirrups at an interval of 200mm c/c. (Award 1 mark for each stirrup) | 3 |  |
|  | Tightened the three stirrups to the horizontal bar with binding wire. (Award 2 marks for each stirrup) | 6 |  |
|  | Concreted the lintel as per the drawing. (Award 2 marks for the formwork and 2 marks for placing) | 4 |  |
|  | Cleaned up the area around the wall (Award 3 marks for cleaning) | 3 |  |
|  | Dimensions - 600mm - 400mm x 2 - 880mm - 800mm (Award 1 mark for each correct measurements for +2mm or – 2mm) | 2 4 |  |
|  | Joint - Uniform - Thickness - Neatness (Award 1 mark for each) | 3 |  |
|  | Squareness (Award two marks for correct square at any two places) | 4 |  |
|  | Horizontal level (Award two marks for correct level at any two points of the wall) | 4 |  |
|  | TOTAL | 70 |  |
| REMARKS: |  |  |  |

NB: Photos and videos should be taken as the candidate performs items 3, 6, 7, 11, 13 and 14.

SECTION 4: ORAL ASSESSMENT (25 MARKS)

Assessor to award marks for each correct response by the candidate in the table below:

| Q# | Question | Expected Key Points | Max Marks | Awarded |
| --- | --- | --- | --- | --- |
| 1 | #### What PPE is needed when building walls? | Safety boots, gloves, and potentially eye protection during mortar mixing and laying. (Award 1 mark each) | 3 |  |
| 2 | Why do we need to place DPM when constructing ground floor? | To prevent moisture rising from the foundation into walls placed as per drawing instructions. (Award 1 mark each) | 1 |  |
| 3 | #### What should you do before starting the setting-out process of a building? | Clear the site of debris and obstacles, confirm accuracy of survey reference points, and ensure all tools are in proper working condition (Award 1 mark each) | 3 |  |
| 4 | What purpose do profile boards serve in setting out? | Marking Building Lines & Foundation Layout, Controlling Trench Position, Width, and Depth, Supporting String Lines for Alignment (Award 1 mark each) | 4 |  |
| 5 | Mention three ways of preparing trenches before casting concrete? | Clean the trench base of debris, level and compact it as specified by the drawings. This ensures a stable and even foundation bed. (Award 1 mark each) | 3 |  |
| 6 | #### Why do we place blinding layer on the hardcore. | Fill spaces between the hardcore, Preventing DPM from being punctured by hardcore, Minimizing use of more concrete | 3 |  |
| 7 | How do you transfer measurements from the working drawing to the site when constructing foundation walls? | Read the Drawing Carefully, Set Out Base Lines, Use Right-Angle Methods, Measure Offsets, mark the position of the wall. | 4 |  |
| 7 | Why do we need to cure concrete during construction work | To maintain moisture, prevents rapid drying or temperature changes that can cause cracks, to improve strength, to reduce shrinkage cracks, to enhance durability | 4 |  |
|  | Total Oral |  | 25 |  |

SUMMARY OF ASSESSMENT

| SECTION | Total Marks | Marks Awarded |
| --- | --- | --- |
| Assessment 1 | 50 |  |
| Assessment 2 | 60 |  |
| Assessment 3 | 70 |  |
| TOTAL | 135 |  |
| Percentage % |  |  |
| ORAL ASSSESMENT |  |  |
| Oral assessment | 25 |  |
| Percentage % |  |  |

ASSESMENT OUTCOME The candidate was found to be: Competent Not yet Competent (Please tick as appropriate) The candidate is competent if the candidate obtains at least 50%

THIS IS THE LAST PRINTED PAGE