MASONRY

CON/OS/MSN/02/4/MA

## PERFORM MASONRY WORK FINISHES

MAY-JULY 2025

TVET CURRICULUM DEVELOPMENT, ASSESSMENT AND CERTIFICATION COUNCIL (TVET CDACC)

PRACTICAL ASSESSMENT

INSTRUCTIONS TO THE ASSESSOR:

- This assessment is to take place in the prescribed order as arranged in the tool.

- Capture clear photographs and/or videos of each candidate’s work at critical points as they perform the tasks and label all media files with: Candidate Registration Number, Unit Code, Practical Session Number, and Date.

- Record candidate scores and assessor remarks in the observation checklists for each session.

Store all completed checklists, media files, and candidate drawings in a secure digital/physical folder per candidate

- CANDIDATE & ASSESSOR DETAILS

| Candidate Name: | CDACC Reg. No.: |
| --- | --- |
| Assessor Name: | Assessor ID Number: |

- PROJECT BRIEF

In this project, the candidate will be required to demonstrate competence in all three elements by completing three hands-on sessions (each ~4 hours) and an oral assessment (~1 hour). Follow the working drawings and specifications provided.

- SECTION 1 (SESSION 1: 4 HOURS): WALL PLASTERING

| Assessment Date: |  |
| --- | --- |
| Assessment Venue: |  |

Elements Covered:

- Carry out wall plastering

Task

The candidate should:

- Construct a 3-course masonry wall in stretcher bond of length 1500mm.

- Apply 20mm plaster to one face of the wall using a 1:3 lime-sand mix.

- Provide a 100mm mortar skirting on the plastered face (25mm).

PRACTICAL CHECKLIST

| No. | Items to Evaluate | Max marks | Marks Awarded |
| --- | --- | --- | --- |
|  | Wore PPEs - Overall - Gloves - Safety Boots - Safety Helmet (Award 1 for each PPE worn) | 4 |  |
|  | Assembled all the required tools. - Tape measure - Trowel - Shovel - Wood float - Spirit level - Plumb bob - Straight edge - Masons hammer (Award 1mark x 6 for correctly assembled) | 8 |  |
|  | Assembled all the required Materials. - Block - Lime - Sand (Award 1 marks for the assembled materials) | 3 |  |
|  | Mixed the mortar for plastering (Award marks for ratio used in mixing and 1 mark for the correct mixing procedure) | 2 |  |
|  | Preparation of the wall to receive plastering - Hacking - Removal of dust particles - Wetting the surface (Award 1 mark for each procedure) | 3 |  |
|  | Plastered the wall - Roughing of the surface - Alignment of the surface - Final finishing of the surface (Award 2 marks for each for each step) | 6 |  |
| 7. | Smooth finished surface (Award 4 marks for smooth and evenly distributed plaster) | 4 |  |
| 8. | Uniform thickness of plaster. (Award 2 marks for uniformity) | 2 |  |
| 9. | Plumbness of plaster. (Award 1 mark each for any 3 places) | 3 |  |
| 10. | Appearance (Award 2 marks for good appearance) | 3 |  |
| 11. | Completeness 100% - 4 75% - 3 50% - 2 Below 50% - 1 | 4 |  |
|  | TOTAL | 42 |  |

NB: Photos and videos should be taken as the candidate performs items  5 and 6.

- SESSION 2 (SECTION 2: 4 HOURS): SCREEDING

| Assessment Date: |  |
| --- | --- |
| Assessment Venue: |  |

Element Covered:

- Lay floor screed

TASK

The candidate should:

- Prepare a 1m2 floor surface for screed. (clean, wet)

- Lay a 50mm thick bonded screed (1m2 Area)

- Level and apply a steel finish

PRACTICAL CHECKLIST

| No. | Items to Evaluate | Max Marks | Awarded Marks |
| --- | --- | --- | --- |
|  | Wore PPEs - Overall - Gloves - Safety Boots - Safety Helmet (Award 1 for each PPE worn) | 4 |  |
|  | Correctly assembled tools. - Tape measure - Trowel - Shovel - Wood float - Spirit level - Straight edge (Award 1mark for correctly assembled tool) | 6 |  |
|  | Correctly assembled materials - Lime - Sand (Award 1 mark for correctly assembled materials for the works) | 2 |  |
|  | Correct mixing of screed. (Award 3 marks for correct mixing or zero) | 3 |  |
|  | Surface preparation to receive the screed. (Award 3 marks for the correct procedure) | 3 |  |
|  | Leveling of the screed (Award 3 marks for the correct procedure) | 3 |  |
|  | Uniform thickness of the screed (Award 2 marks for uniform thickness or zero) | 2 |  |
|  | Screed smooth finish. (Award 3 marks for smoothness or zero) | 3 |  |
|  | Proper housekeeping (Award 3 marks if the area around the project is cleared or zero) | 3 |  |
|  | Appearance (Award 3 marks for good appearance or zero) | 3 |  |
|  | Completeness. 100% - 4 75% - 3 50% - 2 Below 50% - 1 | 4 |  |
|  | TOTAL | 36 |  |

NB: Photos and videos should be taken as the candidate performs items 4, 5 and 6.

- SESSION 3 (SECTION 3: 5 HOURS): KEYING AND POINTING

| Assessment Date: |  |
| --- | --- |
| Assessment Venue: |  |

Element Covered:

- Carry out wall keying and pointing

Task

The candidate should:

- Prepare mortar (1:3 mix) for pointing.

- Erect 4 courses and 1500mm long walling.

- Demonstrate keying on the walling.

- Execute recessed pointing on two lines of the courses.

PRACTICAL CHECKLIST

| No. | Items to Evaluate | Max Marks | Marks Awarded |
| --- | --- | --- | --- |
|  | Wore PPEs - Overall - Gloves - Safety Boots - Safety Helmet (Award 1 Marks for PPE for any 3 worn) | 3 |  |
|  | Assembled and used all tools. - Tape measure - Trowel - Shovel - Wood float - Spirit level - Straight edge (Award 1mark x 6 assembled and used = 6 Marks) | 6 |  |
|  | Mixing of mortar (Award 3 marks for well mixing or zero) | 3 |  |
|  | Wall surface well prepared (Award 3 marks for well-prepared surface or zero) | 3 |  |
|  | Levelness of keyed joints (Award 1 mark each for the 3 courses = 3 Marks0 | 3 |  |
|  | Key pointing (Award 1mark each for the 3 courses 3 Marks) | 3 |  |
|  | Curing and tool cleaning observed (Award 3 marks or zero) | 3 |  |
|  | Proper housekeeping (Award 2 marks if the area around the project is cleaned/ cleared or zero) | 2 |  |
|  | Quality appearance (Award 2 marks for good appearance or zero) | 2 |  |
|  | Completeness. 100% - 4 75% - 3 50% - 2 Below 50% - 1 | 4 |  |
|  | TOTAL | 32 |  |

NB: Photos and videos should be taken as the candidate performs items 3, 4 and 5.

- SECTION 4 – ORAL ASSESSMENT (25 MARKS)

Assessor to Award marks for each correct response by the candidate in the table below:

| No | Question | Expected Answer | Max Marks | Awarded |
| --- | --- | --- | --- | --- |
| 1 | Why is plumbness important in wall plastering? | Ensures vertical alignment, aesthetic finish | 3 |  |
| 2 | What PPE is necessary when applying screed? | Helmet, gloves, goggles, boots | 3 |  |
| 3 | State two types of screeds. | Bonded, unbonded, floating | 3 |  |
| 4 | Why should wall surfaces be dampened before rendering? | Prevent moisture loss from render | 3 |  |
| 5 | What tools are used in pointing? | Pointing trowel, jointing tools | 3 |  |
| 6 | How do you ensure even plaster thickness? | Use dot guides and straight edge | 3 |  |
| 7 | What curing method is used for plaster? | Water spray, damp hessian, plastic sheet | 3 |  |
| 8 | Name one defect caused by improper plaster application. | Cracks, peeling, uneven surface | 4 |  |
|  | TOTAL: |  | 25 |  |

SUMMARY OF ASSESSMENT

| Section | Max Marks | Marks Awarded |
| --- | --- | --- |
| Assessment 1 | 42 |  |
| Assessment 2 | 36 |  |
| Assessment 3 | 32 |  |
| Total | 110 |  |
| Percentage (%) |  |  |
| ORAL ASSESSMENT |  |  |
| Oral (Section 4) | 25 |  |
| Percentage (%) |  |  |

The candidate was found to be: Competent Not yet Competent (Please tick as appropriate) The candidate is competent if the candidate obtains at least 50%